CD-ROM/DVD-ROM DRIVE UNIT

INSTRUCTION MANUAL

REV. 2.1

Interface: IDE/ATAPI

FCC

DECLARATION OF CONFORMITY

PRODUCT NAME: CD-ROM/DVD-ROM DRIVE

MODEL NUMBER:  (CD-ROM) 322D, 361D, 362D, 400D, 401D, 450D, 452D, 480D

(DVD-ROM) DM-822D

FCC RULES: TESTED TO COMPLY WITH FCC PART 15, CLASS B

OPERATING ENVIRONMENT: FOR HOME OR OFFICE USE

FCC COMPLIANCE STA TEMENT:

This device complies with part 15 of the FCC Rules. Operation is subject to the following two conditions: (1) This device may not cause harmful interference, and (2) this device must accept any interference received, including interference that may cause undesired operation.

INFORMATION TO USER:

This equipment has been tested and found to comply with the limits of a Class digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occurred a particular installation, if this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

1. Reorient / Relocate the receiving antenna.

2. Increase the separation between the equipment and receiver.

3. Connect the equipment into an outlet on a circuit difference from that to which

the receiver is connected.

4. Consult the dealer or an experience video/TV technician for help.

CAUTION: Changes or modifications not expressly approved by the manufacturer responsible for compliance could void the users anthority to operate the equipment.

CONTENTS

1. INTRODUCTION

1.1 FEATURES

1.2 POWER SAVE MODE

1.3 SAFETY NOTICE

2.PRECAUTIONS FOR INSTALLATION, OPERATION AND HANDLING

2.1 INSTALLATION PRECAUTIONS

2.2 OPERATION PRECAUTIONS

2.3 HANDLING PRECAUTIONS

3.  PARTS AND FUNCTIONS

3.1 FRONT PANEL

3.2 FUNCTION KEY DESCRIPTION

3.3 REAR PANEL

4.  SPECIFICATIONS

4.1 PERFORMANCE

4.2 ERROR RATES

4.3 AUDIO PERFORMANCE

4.4 GENERAL SPECIFICATIONS

4.5 ENVIRONMENTAL CONDITIONS

5. HARDWARE INSTALLATION

5.1 CD-ROM/DVD-ROM DRIVE INSTALLATION

5.2 CABLE INSTALLATION

6. GUIDE TO OPERATION

6.1  PREREQUISITE FOR INSTALLATION

6.2  CD-ROM/DVD-ROM DEVICE DRIVER  INSTALLATION

6.3  DEVICE DRIVER  (ATAPICD.SYS) PARAMETER LIST

6.4  MICROSOFT CD-ROM EXTENSIONS PARAMETERS

6.5 WINDOWS 95/98, NT & OS2 WARP

7.  MAINTENANCE & TROUBLE SHOOTING

7.1 COMMONLY ENCOUNTERED PROBLEMS AND THEIR SOLUTIONS

1. INTRODUCTION

1.1 FEATURES

. 4.8Mbytes/s (321D, 322D), 5.4Mbytes/s (361D,  362D), 6.0Mbytes (400D, 401D),

6.6Mbytes/s (450D, 452D), 7.5Mbytes/s 2% (480D) maximum data transfer rate.

. 6.75Mbytes/s for DVD, 3.6Mbytes/s for CD maximum data transfer rate (DM-822D)

. Half height, 5.25 inch form factor.

. Motorized tray loading, Open/Close  is  controlled by the EJECT button or by a software command or mechanical eject.

. CD-DA,  CD-ROM/XA,  Photo-CD (Single and Multi-Sessions), CD-I/VIDEO CD,CD Plus

Compatible.

. DM-822D compatible with DVD-ROM (single and dual layer).

. Audio  playback  capability  --  controlled by a software utility or front panel key.

. MPC level 3 compliant.

. Head phone connector on front panel with volume control.

1.2 POWER SAVE MODE

. When the drive unit waits for a command from the host for more than five minutes, it enters the power save mode. The spindle motor and laser stop.

. The drive unit restarts automatically when a host command is received.

1.3 SAFETY NOTICE

. The CD-ROM/DVD-ROM drive should not be exposed to water or other liquids in order to avoid the danger of electric shock.

. The cover of the CD-ROM/DVD-ROM drive protects the user from electric shock and the invisible laser radiation. Do not attempt to remove this cover. There are no user serviceable parts inside.

2.PRECAUTIONS FOR INSTALLATION, OPERATION AND HANDLING

2.1 INSTALLATION PRECAUTIONS

1. Do not install the CD-ROM/DVD-ROM drive in a place where the temperature may exceed the maximum specified operating temperature.

2. Avoid excessively dusty or polluted environments which would tend to make the pickup head lens dirty.

2.2 OPERATION PRECAUTIONS

1. To avoid error, do not move the CD-ROM/DVD-ROM drive during operation.

2. Do not operate the CD-ROM/DVD-ROM drive in a high temperature, high humidity environment.

3. When placing a disc in the tray, be careful not to touch or scratch its surface.

2.3 HANDLING PRECAUTIONS

1. Before moving the CD-ROM/DVD-ROM drive, make sure that the disc has been removed and the tray is closed.

2. To avoid contamination of disc surfaces, always handle discs by their edges.

3. Never bend a disc or expose it to direct sunlight.

3.  PARTS AND FUNCTIONS

3.1 FRONT PANEL

(1) EARPHONE JACK - Accepts a  3.5mm stereo mini plug.

(2) VOLUME - Adjusts the audio output level.

(3) BUSY INDICATOR - Lights up when data is being accessed from a disc, or when the drive is playing audio.

(4) TRAY - Opens and closes for disc loading and unloading.

(5) PLAY/NEXT TRACK BUTTON - Plays a audio CD or  skips to the next track of a audio CD. (Optional)

(6) STOP/EJECT BUTTON - Stops playing a audio CD or eject the tray in/out.

(7) MECHANICAL EJECT - support a 3rd way to eject the tray, such as during power off.

3.2 FUNCTION KEY DESCRIPTION

(1) PLAY / NEXT TRACK BUTTON (Optional)

The PLAY/NEXT TRACK BUTTON can play a CD audio disc or skip to the next audio track.

When the drive is ready and a CD audio disc is inserted, pressing the button to play a CD audio .

When the drive is playing a CD audio, pressing the button to skip to the next audio track.

(2) STOP / EJECT BUTTON

The STOP/EJECT BUTTON can stop playing CD audio or make the tray open or close.

When a CD audio disc is playing, pressing the button to stop the audio playing.

When the drive is in stop mode, pressing the button to open or close the tray.

* Note: If the playing CD audio is initial by application program (Windows media play utility ), pressing STOP/EJECT BUTTON will eject the tray immediately.

3.3 REAR PANEL

(1) AUDIO LINE - DIGITAL OUT CONNECTOR - Digital output format of audio data for

low noise transmission.

(2) AUDIO LINE - ANALOG OUT CONNECTOR - Two channels, left and right audio outputs.

(3) DRIVE MODE SELECT JUMPERS - To determine the drive mode (Master or Slave)

Note : The factory default is the Slave mode.

(4) INTERFACE CONNECTOR - For connecting to host adapter card with 40-conductor data cable.

(5) DC(IN) CONNECTOR - To be connected to a standard power supply.

4.  SPECIFICATIONS*

4.1 PERFORMANCE

(1) Disc diameter		12cm, 8cm

(2) Disc capacity		CD-ROM : 660MB (mode 1, 12cm discs)

DVD-ROM : 4.7GB, 8.5GB

(3) Data transfer rate	4.8MB/s (321D, 322D), 5.4MB/s (361D, 362D),

6.0MB/s (400D, 401D), 6.6MB/s (450D, 452D), 7.5MB/s 2% (480D)

6.75MB/s for DVD, 3.6MB/s for CD (DM-822D)

(4) Access time (Typical)**

1/3 stroke average 	100ms (321D, 322D), 95ms (361D, 362D),

85ms (400D, 401D), 85ms (450D, 452D), 80ms (480D)

150ms for DVD, 90ms for CD (DM-822D)

Random		110ms (321D, 322D), 105ms (361D, 362D),

90ms (400D, 401D), 90ms (450D, 452D), 85ms (480D)

150ms for DVD, 90ms for CD (DM-822D)

(5) Buffer size		128KB (DM-822D is 256KB)

* Specifications are subject to change without notice.

** Average of 200 access in both directions between 15-min 00-sec 00-block and 35-min 00-sec 00-block with latency and layer error correction time.

4.2 ERROR RATES

4.3 AUDIO PERFORMANCE

4.4 GENERAL SPECIFICATIONS

4.5 ENVIRONMENTAL CONDITIONS

* Specifications are subject to change without notice.

5. HARDWARE INSTALLATION

Before beginning installation, please check to make sure that all of the following items are included with your CD-ROM/DVD-ROM package.

1. CD-ROM/DVD-ROM drive unit

2. Installation diskette

3.Audio cable

4. Screws

5.1 CD-ROM/DVD-ROM DRIVE INSTALLATION

1. Remove the cover from an unused 5.25" half-height drive bay on the computer and remove the screws and brackets from both sides of the drive bay.

2. Slide the CD-ROM/DVD-ROM drive into the slot and secure the drive in place either with the screws and brackets from your computer or the mounting screws provided with this drive.

5.2 CABLE INSTALLATION

1. Connect the 40-conductor data cable to the CD-ROM/DVD-ROM drive and IDE/ATAPI

interface card.

2. Plug the power supply cable into the CD-ROM/DVD-ROM drive 4-pin white connector.

6. GUIDE TO OPERATION

This CD-ROM device driver software is compatible with MS-DOS versions 3.1 or later. The installation diskette contains the software needed to install and operate the CD-ROM/DVD-ROM drive.

INSTALL.EXE		Auto installation software for CD-ROM/DVD-ROM drive setup.

ATAPICD.SYS		CD-ROM/DVD-ROM device driver.

*.DOC			Window's WordPad file (WORD 6.0 Format) for detail information.

6.1  PREREQUISITE FOR INSTALLATION

For user with MS-DOS version under 6.0

1. Contact local proper representative to obtain a copy of MSCDEX.EXE file (version 2.22 or above).

2. Copy the MSCDEX.EXE file onto the DOS directory (ex: C:\DOS)

6.2  CD-ROM/DVD-ROM DEVICE DRIVER  INSTALLATION

1.  Insert the installation diskette in drive A or B.

2.  Change the prompt to A:\> or B:\>.

3.  Type INSTALL and press ENTER.

4.  The following display will be shown.

5.  Input the directory name which contains MSCDEX.EXE. Then the following display will be shown.

6.  Follow the instructions and complete the directory choice by pressing ENTER.  At this point, the installation procedure has been prepared and you will see the following display.

If you select YES then press ENTER, the INSTALL program will add an extra command line

DEVICE=C:\CDROMDRV\ATAPICD.SYS /D:CD003 to your CONFIG.SYS file and  a new command line C:\CDROMDRV\MSCDEX /V /D:CD003 /M:10 to your AUTOEXEC.BAT file.  After the modification

procedure is completed, the final display below will be shown.

If, instead,  you select NO, this display will be shown.

Press ENTER to exit. After this modification procedure, it is necessary to reboot your computer.

6.3  DEVICE DRIVER  (ATAPICD.SYS) PARAMETER LIST

The device driver is installed with an entry in CONFIG.SYS.  The syntax of this entry is:

DEVICE=ATAPICD.SYS[ /D:<device_names>][ /P:<port address>] [ /I:<IRQ number>][ / W:16|32]

where the arguments are:

/D:<device_names> The character device names that will be used by MSCDEX.EXE to find and communicate with  the device driver (default is "CD003").

/P:<port address> The port address to be used to communicate with the card.  If this parameter is omitted, the device driver will scan port address automatically during initialize process.

/I:<IRQ number> The IRQ number to be used by the IDE card.  If this parameter is omitted, the device driver will set the IRQ number to default value according the port address.  If the port address is 1F0h (primary IDE), the default IRQ number is 14.  If the port address is 170h (secondary IDE), the default IRQ number is 15.

/W:16|32  Select bus width as 16 or 32 bits.  Default will auto detect IDE-Card ability.

6.4  MICROSOFT CD-ROM EXTENSIONS PARAMETERS

Microsoft CD-ROM Extension (MSCDEX.EXE) is an executable program for MS-DOS (version 3.1 or higher) that works in conjunction with the device driver to allow your computer to access CD-ROM discs in the CD-ROM drive as if they were MS-DOS formatted disks.  This program is written by Microsoft and is specifically designed to work with the Microsoft designed operating system commonly called MS-DOS.

Computers with MS-DOS 'compatible' operating systems may experience unpredictable results unless the operating system manufacturer provides an alternative to the MSCDEX program.

The setup program automatically modifies the autoexec.bat file so that your computer can access the CD-ROM drive.  However, there are some parameters that can be changed if you wish.  This section explains the function of each parameter.  Use a text editor, such as MS-DOS 'EDLIN' or 'EDIT', to modify the appropriate line in your AUTOEXEC.BAT file.

The syntax of the MSCDEX.EXE entry in AUTOEXEC.BAT is :

[drive:][path] MSCDEX.EXE /D:device name[/M:n][/E][/V] [/L:drive letter]

Note : The actual MSCDEX.EXE command line in the AUTOEXEC.BAT file is not truncated and should be placed entirely on one line.

[drive:] [path] MSCDEX.EXE

Specifies the location (drive and directory) of the MSCDEX.EXE file.

The default setting is the CDROMDRV directory of the boot drive.

/D: device name

Specifies the name of the CD-ROM drive (8 characters maximum).

This must be identical to the device name specified in the device driver in the CONFIG.SYS file.  For example :

C:\CDROMDRV\MSCDEX.EXE /D:CD003

[/M:n]

Specifies the number of sector buffers for temporary storage of the most recent disc data.  For example to specify four sector buffers you would use:

C:\CDROMDRV\MSCDEX.EXE  /D:CD003  /M:4

The installation program will set this value to 10. The default value is 4.

Note :The greater this value, the better the CD-ROM drive will perform.  However, each buffer uses about 2k of memory and specifying too many buffers may slow down computer operation or interface with other programs which have large memory usage requirements.  Using expanded memory (see the /E option in this section) or loading MSCDEX.EXE into high memory (refer to your MS-DOS User's Manual or Memory Manager User's Manual) may allow you to specify a larger number of buffers to enhance CD-ROM performance and operate programs with large memory usage requirements simultaneously.

[/E]

Instructs the computer to use expanded memory, if available.  For example :

C:\CDROMDRV\MSCDEX.EXE /D:CD003 /E

Note:That you must first load an expanded memory driver before using this option.  If no expanded memory driver is loaded, the following error message appears:

Expanded memory not present or not usable

6.5 WINDOWS 95/98, NT & OS2 WARP

Please refer to Windows 95/98, NT & OS2 WARP installation guide.

Note: During installation of OS2 operating system, please select "Non-listed IDE CD-ROM" to install the ATAPI IDE interface CD-ROM. Otherwise, the drive can't be detected by the operating system.

7.  MAINTENANCE & TROUBLE SHOOTING

7.1 COMMONLY ENCOUNTERED PROBLEMS AND THEIR SOLUTIONS

If the CD-ROM/DVD-ROM drive does not operate properly, check to see:

1. Are both ends of all the cables securely attached?  Are any of the cables frayed or damaged?

2. Is the disc inserted correctly?  The label must be facing up.

3. If any other hardware add-on cards are installed in the computer, are they using the same address port as the CD-ROM/DVD-ROM drive?  If so, there will be a hardware conflict which you can solve by reconfiguring the address port of the CD-ROM/DVD-ROM drive.

If no sound comes from the speakers:

1. Have the connections between the LINE OUT jacks and the input terminals of the amplifier and/or speakers been properly made?

2. Is power to the amplifier and/or speakers turned on?

If no sound comes form headphones:

1. Are the headphones properly connected to the headphone jack?  Is the headphone cord frayed or damaged?

2.  Is the volume adjustment (on the front panel) set properly?